Development of Regional Impact #1950

Gwinnett Stadium Mixed-Use

Gwinnett County, Georgia

Prepared for:

Brand Properties

Prepared by:

Table of Contents

- Project Description	1

  - Introduction	1

  - Site Plan Review	1

  - Site Access	7

  - Bicycle and Pedestrian Facilities	9

  - Transit Facilities	9

- Traffic Analysis Methodology and Assumptions	11

  - Growth Rate	11

  - Traffic Data Collection	11

  - Detailed Intersection Analysis	14

- Study Network	16

  - Gross trip Generation	16

  - Trip Distribution	16

  - Level of Service Standards	17

  - Study Network Determination	17

  - Existing Facilities	19

- Trip Generation	23

- Trip Distribution and Assignment	24

- Traffic Analysis	36

  - Existing Traffic	36

  - 2012 No-Build Condition Traffic	44

  - 2012 Build Condition Traffic	57

  - Queue Analysis	72

- Identification of Programmed Projects	77

- Ingress/Egress Analysis	79

- Internal Circulation Analysis	80

- Compliance with Comprehensive Plan Analysis	81

- DRI Review Checklist (Non-Expedited Review Criteria)	83

  - General Information	83

  - Regional Policies and Adopted Plans	84

  - Environmental	87

  - Project	90

  - Non-Expedited Transportation Analysis	99

  - ARC Score Sheet	101

List of Figures

Figure	Title				Page

Figure 1     DRI Site Plan		2

Figure 2     Project Location Map		3

Figure 3     Study Area Zoning Classifications	5

Figure 4	Aerial Photography		6

Figure 5	Access Locations		8

Figure 6	Level of Service Definitions and Criteria	15

Figure 7	Functional Classifications		20

Figure 8	Residential/Non-Residential Trip Distribution	25

Figures 9 thru 13

Weekday Project Generated Traffic	26 thru 30

Figures 14 thru 18

Saturday Project Generated Traffic	31 thru 35

Figure 19	Existing (2008) Traffic Conditions	37

Figures 20 thru 22

Existing (2008) Weekday Peak Hour Traffic Volumes	38 thru 40

Figures 23 thru 25

Existing (2008) Saturday Peak Hour Traffic Volumes	41 thru 43

Figure 26	Future (2012) No Build Traffic Conditions	48

Figures 27 thru 30

Future (2012) No-Build Weekday Peak Hour Traffic Volumes	49 thru 52

Figures 31 thru 34

Future (2012) No-Build Saturday Peak Hour Traffic Volumes	53 thru 56

Figures 35 thru 39

Future (2012) Build Weekday Peak Hour Traffic Volumes	58 thru 62

Figures 40 thru 44

Future (2012) Build Saturday Peak Hour Traffic Volumes	63 thru 67

Figure 45	Future (2012) Build Traffic Conditions	70

Figure 46	Programmed Transportation Improvements	78

Figure 47	Study Area Future Land Use Classifications	82

List of Tables

Table	Title				Page

Table 1	Proposed Land Uses		4

Table 2	Gross Trip Generation Analysis	16

Table 3	Presumptive Impact/Significance Threshold	18

Table 4	Functional Classifications		19

Table 5	Net Trip Generation Analysis	23

Table 6	Existing (2008) Intersection Level of Service Summary	36

Table 7	Future (2012) No-Build Condition Intersection Level of Service Summary	46

Table 8	Future (2012) Build Condition Intersection Level of Service Summary	68

Table 9	Queue Analysis		72

Table 10	Programmed Transportation Improvements	77

Appendices

Data Collection	A

Level of Service Worksheets	B

Trip Generation Worksheets	C

Programmed Transportation Improvements	D

Executive Summary

This report provides a study of the anticipated traffic impacts that are associated with the construction of a mix-use development that envelops the recently approved “Gwinnett Minor League Baseball Stadium, DRI #1725, in Gwinnett County, Georgia.  The proposed project is located just south of Interstate 85 (I-85) along State Route 20 (SR 20) between Rock Springs Road and Old Peachtree Road.  This project will likely include a mixture of uses consisting of attraction, retail, office and residential land uses.  The proposed development has been estimated to be completed in a single construction phase by the year 2012.

The applicant’s proposal is for a mixture of uses that encompass the Gwinnett County Minor League Baseball Stadium.  Access for the development has been proposed at nine locations.   There are three access locations along SR 20, one serving as full access, while the remaining two will be constructed as right-in/right-out driveways.  The full access location along SR 20 will be directly aligned with an existing median opening.  The remaining access locations associated with this proposed development will be constructed along Rock Springs Road, Tech Center Parkway, Tech Center Drive and Old Peachtree Road.  Each of the access locations will be shared among each use and will work to disperse the project generated traffic within the proposed development, as well as, throughout the established study area.  This proposed development will supplement the minor league baseball stadium that is currently under construction providing additional employment, housing and recreational opportunities to Gwinnett County.  There are 617,000 square feet of office space, 351,000 square feet of retail space, 300 hotel rooms and 610 residential units planned to be constructed on approximately 73.50 acres.

Pursuant to GRTA’s request, the traffic study for this project entailed a detailed intersection analysis for the following intersections:

- SR 20 @ Russell/Ridge Rd

- SR 20 @ Azalea Dr (south of Bentbrooke Trl)

- SR 20 @ Bentbrooke Trl

- SR 20 @ Azalea Dr (north of Bentbrooke Trl)

- SR 20 @ Old Peachtree Rd

- SR 20 @ Tech Center Pkwy

- SR 20 @ Rock Springs Rd

- signalized intersection between I-85 and Rock Springs Rd

- SR 20 @ Mall of Georgia Blvd

- SR 20 @ Plains Court Wy

- SR 20 @ Mall Grand Ln

- SR 20 @ Woodward Crossing Blvd

- SR 20 @ Gravel Springs Rd

- Old Peachtree Rd @ Horizon Drive

- Old Peachtree Rd @ Collins Hill Rd.

- Old Peachtree Rd @ Rock Springs Rd (west of SR 20)

- Old Peachtree Rd @ Tech Center Pkwy

- Old Peachtree Rd @ Rock Springs Rd (east of SR 20)

- Old Peachtree Rd @ Braselton Highway (SR 124)

- Braselton Highway (SR 124) @ Gravel Springs Rd (SR 324)

- Tech Center Pkwy @ Tech Center Dr

- Rock Springs Rd @ Tech Center Dr

- All proposed access locations

For the purposes of this analysis, the proposed development was found to produce 18,280 vehicles trip ends per weekday and 16,894 vehicle trip ends on Saturday.  The total gross trips generated by the proposed development were adjusted to account for the mixture of uses.  Additional adjustments were made to account for trip reductions that result from alternative modes of travel and for any pass-by trips associated with the commercial component of this development.  These trips were then distributed and assigned to the study area.  Detailed capacity analyses of the intersections studied in this report were performed using the modeling software CORSIM, version 5.1 utilizing peak hour traffic volumes for the weekday AM and PM peak hours and for the Saturday peak hour.

The results from the detailed intersection analysis for the future (2012) No-Build Condition, which does not include the traffic generated by the proposed development, and the future (2012) Build Condition indicate that roadway improvements will be required to serve both the general growth in background traffic and the traffic associated with the proposed development.  These improvements are necessary in order for all of the intersections, identified previously, to operate at an acceptable level of service (LOS).

The improvements recommended for the future (2012) No-Build Conditions are as follows:

- Widen SR 20 from four (4) to six (6) lanes from Tech Center Pkwy to I-85

- SR 20 @ signalized intersection between Rock Springs Rd and I-85

  - Provide a southbound right-turn bay along SR 20

- SR 20 @ Rock Springs Rd

  - Provide a westbound, right-turn bay along Rock Springs Rd

- SR 20 @ Azalea Dr (north of Bentbrooke Trl)

  - Signalize intersection

  - Provide an eastbound, right-turn bay along Azaela Dr

- SR 20  @ Azalea Dr (south of Bentbrooke Trl)

  - Provide eastbound, right and left-turn bays along Azalea Dr

  - Provide westbound, right and left-turn bays along Azalea Dr

- SR 20 @ Russell/Ridge Rd

  - Provide an eastbound, left-turn bay along Russell Rd

  - Provide a westbound, left-turn bay along Ridge Rd

- Old Peachtree Rd @ Rock Springs Rd (east of SR 20)

  - Signalize intersection

  - Provide a southbound, right-turn bay along Rock Springs Rd

  - Provide and eastbound, left-turn bay along Old Peachtree Rd

The improvements recommended for the future (2012) Build Conditions are as follows:

- Widen SR 20 from four (4) to six (6) lanes from Azalea Dr (north of Bentbrooke Trl) to Tech Center Pkwy

- SR 20 @ Tech Center Pkwy

  - Provide a southbound right-turn bay along SR 20

- SR 20 @ Old Peachtree Rd

  - Extend the eastbound, left-turn bay along Old Peachtree Rd

  - Convert existing southbound right-turn bay to a through movement and provide a southbound right-turn bay along SR 20

- SR 20 @ Azalea Dr (south of Bentbrooke Trl)

  - Signalize intersection

- Old Peachtree Rd @ Collins Hill Rd

  - Extend eastbound right-turn bay

The following table provides a general summary for the DRI Non-Expedited Review:

| Name and Number of DRI | Gwinnett Stadium Mixed-Use DRI #1950 |
| --- | --- |
| Jurisdiction | Gwinnett County |
| Local Development Approval Sought | Rezoning |
| Location | West of State Route 20 (Buford Drive) between Old Peachtree Road and Rock Springs Road |
| Uses and Intensities | 617,000 sq ft of office, 351,000 sq ft of retail, 300 hotel rooms and 610 multi-family residential units |
| Project Phasing and Build Out Schedule | Single Phase for the year 2012 |
| Gross Trip Generation (ADT/AM Peak/PM Peak) | 26,115 /1,515 /2,646 |
| Saturday Trip Generation | 25,905/2,405 |

- Project Description

  - Introduction

The following analysis reports the anticipated traffic impacts that are associated with the construction of a mix-use development that envelops the recently approved “Gwinnett Minor League Baseball Stadium, DRI #1725.  The applicant’s intent is to construct a well balanced mixed-use community that offers additional opportunities for employment, housing and recreation.  This document addresses another phase of construction that will supplement the Gwinnett County Minor League Baseball Stadium that is currently under construction.  The proposed development has been estimated to be completed by the year 2012.

This proposed development has been classified as a Development of Regional Impact (DRI).  A DRI can be defined as a large scale development that is likely to have effects outside of the local government jurisdiction in which they are located.  The Department of Community Affairs (DCA) has established tiers and thresholds for developments based upon which county they are located in.  A county can be classified either as an Atlanta Region, Metropolitan Region or Non-metropolitan Region County.  An exceeded threshold is subject to regional review prior to any local government action as required by state law and DCA rules.

The proposed mixed-use development is located in Gwinnett County, which is classified as an Atlanta Region County; therefore, regional review is required if this development constructs more than 500,000 gross square feet.  Based upon the site plan that is provided in Figure 1, this development requires regional review.  Coordination shall be conducted between the host local government and the appropriate review agencies, which consist of the ARC and the GRTA.  This analysis has been prepared as part of a non-expedited review submittal as a result of submitting a re-zoning application to Gwinnett County, Georgia.

The site for the proposed development is located in the northern portion of Gwinnett County, west of State Route 20 (SR 20) and just south of Interstate 85 (I-85).  The proposed site along SR 20 is situated between Rock Springs Road, to the north, and Old Peachtree Road, to the south.  The location of the subject property has been further illustrated in Figure 2.

  - Site Plan Review

As stated previously, the site is located along SR 20 between Rock Springs Road and Old Peachtree Road.  The site plan is unique to the area in that the mixture of additional land uses, which work to provide places of employment, residence and recreation, surround the site’s principle attraction, a minor league baseball stadium that is affiliated with the Atlanta Braves major league baseball team.  All parking associated with the proposed uses will be constructed as both surface and structured parking.

Figure 1

Figure 2

The proposed development has been scheduled to be completed in one construction phase that has been estimated to be completed by the year 2012.  The site for the subject property is currently zoned with multiple zoning classifications, consisting of M1, OI, C2 and R100, in Gwinnett County, and will require a rezoning process to accommodate the proposed mixture of uses.  The applicant will be seeking a MUO (Mixed-Use Overlay District) zoning classification.  Adjacent properties west of the subject property are zoned as R-140 and M-1.  The R-140 zoning classification consists of a church facility.  Properties north of the subject property are zoned as C2 and R100, while properties south of the subject property are zoned as R100, C1 and C2.  SR 20 is adjacent to the subject property’s eastern property line.  Zoning classifications east of SR 20 include OI and R-100.  These zoning classifications within the vicinity of the subject property have been displayed in Figure 3.

The proposed development anticipates constructing retail, office, multi-family residential and hotel uses on approximately 73.50 acres.  A summary of these land uses and densities are provided in Table 1, below.

| Table 1 |  |
| --- | --- |
| Proposed Land Uses |  |
| Use | Density |
| Multi-Family Residential | 610 units |
| Office | 617,000 sq ft |
| Commercial/Retail | 351,000 sq ft |
| Hotel | 300 rooms |

Currently, with the exception of residences that have been acquired, the subject property is mostly undeveloped.  There is a multi-family development under construction adjacent to the subject property’s western boundary.  A church facility and light industrial properties adjoin the subject property’s western property line, while retail properties are adjacent to both the northeast and southeast corners of the property.  These existing land uses have been illustrated in Figure 4.

Off-street parking will be provided in accordance with the Gwinnett County zoning ordinances, development guidelines, and tree protection and replacement ordinances. Specifically, the following parking ratios are proposed for the subject property:

- Residential (multi-family):

1.5 parking space per dwelling unit (min)/3.0 parking space per dwelling unit (max)

- Office:

1 parking space per 300 square feet

- Retail:

4.0 parking space per 1,000 square feet

- Hotel:

1.0 parking space per guest room

Figure 3

Figure 4

Parking will be provided in a manner that is conveniently located to the proposed use that it is to serve. All parking spaces will be landscaped to meet the tree replacement and protection standards for Gwinnett County.

  - Site Access

Vehicular access for the proposed development has been proposed at nine locations.  Five of these locations are in addition to the access locations provided by the minor league baseball stadium.  Each location will serve as shared access to surface parking areas and deck parking structures as all uses have been inter-connected with pedestrian facilities.  The locations for these proposed access locations are illustrated in Figure 5.  For the purposes of this analysis the proposed driveways have been identified as Access A through J.  Access D was omitted from this analysis as a recommended condition for DRI #1725, the Gwinnett County Minor League Baseball Stadium.

As one can see in Figure 5, Access locations A and C, along SR 20, will be constructed as right-in/right-out movements, while Access B will be constructed as full-movement location that is directly aligned with an existing median opening.  Access E has been directly aligned with Tech Center Drive and the remaining access locations do not have any opposing driveways.

- Access A serves SR 20 southbound traffic with a single ingress lane to surface area parking and parking deck structures.  Egress traffic from Driveway A will be constructed as a single outbound lane to SR 20 southbound.

- Access B serves both northbound and southbound vehicular traffic along SR 20 and will be constructed as a four-lane driveway accessing the surface parking areas and parking deck structures for the proposed development.  There will be two (2) ingress lanes accommodating a left-turn movement from SR 20 northbound along with the SR 20 southbound right-turn movement.  There will also be two egress lanes; each of these lanes shall be designated as “left-only” turn movements along with a right-turn bay.

- Access C serves SR 20 southbound traffic with a single ingress lane to surface area parking and parking deck structures.  Egress traffic from Driveway C will be constructed as a single outbound lane to SR 20 southbound.

- Access D was omitted from this analysis as a recommended condition for DRI #1725, the Gwinnett County Minor League Baseball Stadium.

- Access E serves as an alternate access to both the surface area parking and the parking deck structures for the proposed development from Tech Center Parkway.  This driveway will be constructed as a four-lane driveway with two (2) ingress lanes and two (2) egress lanes.  The egress lanes for this driveway will consist of a right-only lane and a through lane and will be further supplemented with a left-turn bay.  As stated previously, this driveway has been proposed to be directly aligned with Tech Center Drive.

Figure 5

- Access F serves as access to residential and office uses from Tech Center Drive.  This access location will be constructed with a single ingress lane to surface area and parking deck structures and will contain one egress lane.

- Access G also serves as access to residential and office uses from Rock Springs Road.  This access location will be constructed with a single ingress lane to surface area and parking deck structures and will contain one egress lane.

- Access H serves as access to residential uses from Rock Springs Road.  This access location will be constructed with one ingress lane to surface area parking and will be constructed with one egress lane.

- Access I serves as access to residential and office uses from Tech Center Parkway.  This access location will be constructed with a single ingress lane to surface area and parking deck structures and will contain one egress lane, supplemented with a left-turn bay.

- Access J serves as an alternate access to both the surface area parking and the parking deck structures for the proposed development from Old Peachtree Road.  This driveway will be constructed as a two-lane driveway with one (1) ingress lane and one (1) egress lane.

  - Bicycle and Pedestrian Facilities

Currently, there are no bicycle or pedestrian facilities that exist along SR 20, for the portion of the roadway facility that adjoins the subject property.  There are however, bicycle and pedestrian facilities located along Rock Springs Road, Tech Center Drive, Tech Center Parkway and Old Peachtree Road within close proximity to the subject property.  An eight-foot multi-use path currently exists adjacent to the eastbound travel lane along Tech Center Parkway.

The proposed development that is situated between Old Peachtree Road and Rock Springs Road will work to maintain and enhance the existing system of bicycle and pedestrian facilities by connecting to any existing sidewalks adjacent to the site.  Specifically, access locations G and I provide pedestrian amenities internal to the proposed site that connect Rock Springs Road and Tech Center Parkway.  Access locations B, E and J provide pedestrian amenities that allow pedestrian ingress/egress from Tech Center Parkway, SR 20 and Old Peachtree Road.   The proposed development will be providing newly constructed bicycle and pedestrian facilities within the development for enhanced pedestrian connectivity.

  - Transit Facilities

There are not any rail transit opportunities within close proximity to the proposed development.  There are, however, bus transit operations offered by both Gwinnett County Transit and the Georgia Regional Transportation Authority that operate during the week and on Saturdays.  Facilities for these transit opportunities are located within close proximity to the proposed project.  There is a local bus route that operates during the week and on Saturdays, while Gwinnett County Express Routes and GRTA Xpress Routes are available for weekday commuter trip options.  None of the routes stop within one quarter mile of the proposed development; however, there is a potential for route adjustments once the proposed development is constructed.

The local bus route within the immediate vicinity of the proposed project is “Route 50”.  This local bus service connects the City of Buford to the Gwinnett Transit Center, which is located near the Gwinnett Place Mall.  The transfer points along this route are located at the Gwinnett Place Mall, Discover Mills, the Mall of Georgia and the park and ride lot at the intersection of I-985 and SR 20.  Gwinnett County Transit’s Express Routes 101 and 101A provide commuter options for those within close proximity to the proposed project during the week.  The park and ride lot at the intersection of I-985 and SR 20 serves as the transfer point for Route 101 and this route’s destination is Capital Square in the City of Atlanta.  Express Route 101A utilizes a transfer point located within the Mall of Georgia and has a destination point at MARTA Five Points in the City of Atlanta.  Lastly, there are two GRTA Xpress Routes within close vicinity to the proposed project with separate destinations.  Each GRTA Xpress Route has a transfer point at Discover Mills in the northern portion of Gwinnett County.  Route 410 travels to the MARTA Lindbergh Station, while Route 412 provides a connection to Midtown Atlanta at the Civic Center MARTA Station and the Arts Center MARTA Station.  The proposed development will be providing amenities to encourage transit ridership, such as a bus shelter, potential shuttle service, sidewalks and bicycle racks.

  - Traffic Analysis Methodology and Assumptions

  - Growth Rate

Future background traffic for the proposed development was based upon assumptions provided by GRTA.  For the purposes of this analysis, a four percent (4%) annual growth rate was assumed for the roadways within the study area, with the exception of the following intersections, where a two and one-half percent (2.5%) annual growth rate was assumed:

- Old Peachtree Rd @ Collins Hill Rd

- Old Peachtree Rd @ Rock Springs Rd (east of SR 20)

- Old Peachtree Rd @ Braselton Hwy

- Old Peachtree Rd @ Horizon Dr

- Gravel Springs Rd @ Braselton Hwy

  - Traffic Data Collection

Traffic count data for the proposed project has been obtained from All Traffic Data Services, Inc.  Peak-hour intersection turning movement counts were collected at each intersection identified during the pre-methodology meeting.  These count locations are within the immediate vicinity of the proposed project and were taken for both the AM and PM weekday peak periods and for the Saturday peak periods.  When applicable, those traffic count data that were utilized for DRI #1725 (Gwinnett County Minor League Baseball Stadium) were incorporated into this analysis and adjusted to reflect current conditions.  Peak hour traffic count worksheets are provided in Appendix A.

The traffic count data that was incorporated into this analysis was collected in the year 2008.  Traffic counts that have been utilized from DRI #1725 were collected on February 5th and February 9th, 2008.  The remaining traffic count locations that were not included in the analyses for DRI #1725 were collected on September 3rd and September 6th, 2008.  Both signalized and un-signalized intersections within the study area, along with peak hours of traffic, include:

- SR 20 @ Russell/Ridge Rd (signalized)

  - 8:00 am to 9:00 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 11:30 am to 12:30 pm			Saturday Peak Hour

- SR 20 @ Azalea Dr (south of Bentbrooke Trl – un-signalized)

  - 7:45 am to 8:45 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:15 pm to 1:15 pm				Saturday Peak Hour

- SR 20 @ Bentbrooke Trl (un-signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:00 pm to 1:00 pm				Saturday Peak Hour

- SR 20 @ Azalea Dr (north of Bentbrooke Trl – un-signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:00 pm to 1:00 pm				Saturday Peak Hour

- SR 20 @ Old Peachtree Rd (signalized)

  - 8:00 am to 9:00 am					AM Peak Hour

  - 4:30 pm to 5:30 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- SR 20 @ Tech Center Pkwy (signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 11:45 am to 12:45 pm			Saturday Peak Hour

- SR 20 @ Rock Springs Rd (signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 11:45 am to 12:45 pm			Saturday Peak Hour

- signalized intersection between I-85 and Rock Springs Rd (signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- SR 20 @ Mall of Georgia Blvd (signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:15 pm to 1:15 pm				Saturday Peak Hour

- SR 20 @ Plains Court Wy (signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:15 pm to 1:15 pm				Saturday Peak Hour

- SR 20 @ Mall Grand Ln (signalized_

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:15 pm to 1:15 pm				Saturday Peak Hour

- SR 20 @ Woodward Crossing Blvd (signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:15 pm to 1:15 pm				Saturday Peak Hour

- SR 20 @ Gravel Springs Rd (SR 324 – signalized)

  - 7:45 am to 8:45 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:15 pm to 1:15 pm				Saturday Peak Hour

- Old Peachtree Rd. @ Horizon Drive (signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 4:45 pm to 5:45 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- Old Peachtree Rd. @ Collins Hill Rd (signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- Old Peachtree Rd @ Rock Springs Rd (west of SR 20 – un-signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 4:45 pm to 5:45 pm				PM Peak Hour

  - 12:00 pm to 1:00 pm				Saturday Peak Hour

- Old Peachtree Rd. @ Tech Center Pkwy (un-signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 4:30 pm to 5:30 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- Old Peachtree Rd. @ Rock Springs Rd. (east of SR 20 – un-signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 11:45 am to 12:45 pm			Saturday Peak Hour

- Old Peachtree Rd. @ Braselton Highway (SR 124 - signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- Braselton Highway (SR 124) @ Gravel Springs Rd (SR 324 - signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:30 pm to 1:30 pm				Saturday Peak Hour

- Tech Center Pkwy @ Tech Center Dr (un-signalized)

  - 7:30 am to 8:30 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:00 pm to 1:00 pm				Saturday Peak Hour

- Rock Springs Rd. @ Tech Center Dr (un-signalized)

  - 7:15 am to 8:15 am					AM Peak Hour

  - 5:00 pm to 6:00 pm				PM Peak Hour

  - 12:00 pm to 1:00 pm				Saturday Peak Hour

  - Detailed Intersection Analysis

Capacity analysis procedures for this project involved a detailed intersection analysis.  The capacity analysis procedures for the detailed intersection analysis were performed using the software CORSIM, version 5.1.  CORSIM is a computer model developed by the Federal Highway Administration (FHWA) that simulates a whole hour of traffic and monitors the status of each vehicle as it travels through the network.  CORSIM tabulates the average delay per vehicle at each intersection within the study area that will be used to provide a level of service (LOS) for each approach and intersection.  Levels of service worksheets for this analysis are provided in Appendix B.

The capacity analysis for this project has assumed that LOS D or better will be considered adequate (or acceptable) for the roadways within the study area.   It should be noted that when completing traffic analysis for a project within an urban area, a level of service D or better is considered adequate or acceptable.   Anything worse than a level of service D would indicate that an intersection or approach is approaching capacity and cannot accommodate substantial increases in traffic.  Therefore, if an intersection is found to operate at level of service D, this does not mean that there is a lack of congestion: rather, it would indicate that, as a whole, the intersection could still accommodate additional traffic without breaking down in the peak hours of operation.

LOS is a measure used to describe traffic operations that translates conditions into a letter grade ranging from A to F.  Figure 6, which is based on the Highway Capacity Manual, illustrates and describes each LOS and lists the criteria used in their determination.

Figure 6

  - Study Network

  - Gross Trip Generation

As discussed previously, the proposed development anticipates constructing 610 multi-family residential units, 617,000 square feet of office space, 351,000 square feet of retail space and 300 hotel rooms.  The Institute of Transportation Engineers (ITE) Trip Generation Manual, 7th edition, was utilized to generate project trips for both the weekday AM and PM peak periods and for the Saturday peak period.  The gross trip generation analysis for the proposed development has been illustrated in Table 2 and the trip generation worksheets are provided in the Appendix C.

| Table 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gross Trip Generation Analysis |  |  |  |  |  |  |  |
| Weekday |  |  |  |  |  |  |  |
| Land Use | Density | ITE Code | Daily Trip Ends | AM Peak |  | PM Peak |  |
|  |  |  |  | In | Out | In | Out |
| Residential | 610 units | 230 | 3,027 | 37 | 182 | 178 | 87 |
| Office | 617,000 sq ft | 710 | 5,416 | 708 | 96 | 131 | 639 |
| Commercial/Retail | 351,000 sq ft | 820 | 15,360 | 203 | 129 | 688 | 746 |
| Hotel | 300 rooms | 310 | 2,312 | 98 | 62 | 94 | 83 |
| Total: |  |  | 26,115 | 1,046 | 469 | 1,091 | 1,555 |
| Saturday |  |  |  |  |  |  |  |
| Land Use | Density | ITE Code | Daily Trip Ends | Peak |  |  |  |
|  |  |  |  | In |  | Out |  |
| Residential | 610 units | 230 | 2,633 | 117 |  | 100 |  |
| Office | 617,000 sq ft | 710 | - | - |  | - |  |
| Commercial/Retail | 351,000 sq ft | 820 | 20,122 | 1,027 |  | 948 |  |
| Hotel | 300 rooms | 310 | 3,150 | 119 |  | 94 |  |
| Total: |  |  | 25,905 | 1,263 |  | 1,142 |  |

  - Trip Distribution

Trip distribution and assignment for the proposed project has been based upon an evaluation of population and employment estimates from the latest ARC Travel Demand Model.  The trips associated with the proposed development were categorized for both residential and non-residential trip purposes.  Non-residential trips were based upon population densities, as these types of trips are generated by commercial uses.  The residential trips were based upon employment densities, assuming that residences will travel to places of employment.  The directional distribution and assignment for project generated traffic was agreed upon during methodology discussions with GRTA, ARC and Gwinnett County staff.

  - Level of Service Standards

For the purposes of this analysis, LOS D was used for impact assessment and mitigation analysis.  If, however, an intersection currently operates at LOS E or LOS F during an existing peak period, the LOS standard for that peak period becomes LOS E, consistent with GRTA’s Letter of Understanding.

  - Study Network Determination

A general study area was determined using GRTA’s seven percent (7%) rule.  This method of evaluation identifies roadway segments where the trips generated by the proposed DRI exceed 7% of the two-way, daily service volumes at the appropriate level of service standard.  For this proposed development, the appropriate level of service standard was LOS D.  Roadway facilities that are anticipated to receive an impact greater than 7% include SR 20, Old Peachtree Road, Collins Hill Road, Interstate 85, Rock Springs Road, Tech Center Parkway, Tech Center Drive and a portion of Lawrenceville Suwanee Road.  Table 3 reveals the “Presumptive Impact/Significance Threshold” analysis for the proposed project.  A study area was agreed upon during the methodology meeting with GRTA, ARC, GDOT and Gwinnett County staff and pursuant to GRTA’s recommendations the following intersections are included in this analysis:

- SR 20 @ Russell/Ridge Rd

- SR 20 @ Azalea Dr (south of Bentbrooke Trl)

- SR 20 @ Bentbrooke Trl

- SR 20 @ Azalea Dr (north of Bentbrooke Trl)

- SR 20 @ Old Peachtree Rd

- SR 20 @ Tech Center Pkwy

- SR 20 @ Rock Springs Rd

- signalized intersection between I-85 and Rock Springs Rd

- SR 20 @ Mall of Georgia Blvd

- SR 20 @ Plains Court Wy

- SR 20 @ Mall Grand Ln

- SR 20 @ Woodward Crossing Blvd

- SR 20 @ Gravel Springs Rd

- Old Peachtree Rd @ Horizon Drive

- Old Peachtree Rd @ Collins Hill Rd.

- Old Peachtree Rd @ Rock Springs Rd (west of SR 20)

- Old Peachtree Rd @ Tech Center Pkwy

- Old Peachtree Rd @ Rock Springs Rd (east of SR 20)

- Old Peachtree Rd @ Braselton Highway (SR 124)

- Braselton Highway (SR 124) @ Gravel Springs Rd (SR 324)

- Tech Center Pkwy @ Tech Center Dr

- Rock Springs Rd @ Tech Center Dr

- All proposed access locations

Each of the intersections listed previously were analyzed for the Existing 2008 Conditions, the 2012 No-Build Conditions, and the 2012 Build Conditions.  The time periods analyzed were for the weekday AM and PM peak hours and for the Saturday peak hour.

  - Existing Facilities

Data for the study area roadways was gathered from a Roadway Characteristics Inventory Database (RCI) file obtained from GDOT.  The functional classifications for the roadways within the immediate vicinity of the subject property consist of an interstate principal arterial, minor arterial, major/minor collector and local roadway facilities.  These roadway facilities are illustrated in Figure 7, along with the remaining roadways within the study area.  Table 4 summarizes the description of the major roadways within the vicinity of the project and more detailed descriptions follow.

| Table 4 |  |  |  |
| --- | --- | --- | --- |
| Functional Classifications |  |  |  |
| Roadway | Rural/Urban Designation | Functional Classification | LOS Standard |
| Interstate 85 | Urban | Principal Arterial | D |
| State Route 20 (Buford Dr) | Urban | Minor Arterial | D |
| State Route 324 (Gravel Springs Rd) | Urban | Major Collector | D |
| State Route 124 (Braselton Hwy) | Urban | Major Collector | D |
| Old Peachtree Rd | Urban | Minor Collector | D |
| Rock Springs Rd | Urban | Local Road | D |
| Tech Center Parkway | Urban | Local Road | D |
| Tech Center Dr | Urban | Local Road | D |
| Azalea Dr | Urban | Local Road | D |
| Brentbrooke Trl | Urban | Local Road | D |
| Russell/Ridge Rd | Urban | Local Road | D |
| Horizon Dr | Urban | Minor Collector | D |
| Collins Hill Rd | Urban | Minor Collector | D |

Figure 7

Interstate 85

- Interstate 85 at this location is a four-lane divided principal interstate arterial with a posted speed limit of 65 mph.  Access to SR 20 is provided at exits 115 A and B.  The entrance and exit ramps at SR 20 are directional flow ramps without any traffic control devices.  Interstate 85 is a north/south route across the State of Georgia.

SR 20

- SR 20 is a four-lane divided roadway facility (vegetated median) with turn lanes and a posted speed limit of 50 mph.  SR 20 is classified as a minor arterial that provides connections to multiple cities within Gwinnett County.  In particular, SR 20 provides a connection for the cities of Loganville, Grayson, Lawrenceville and Buford.  SR 20 also serves as a connector to Interstates 85 and 985.  Interstate 85 is located approximately one mile north of the subject property.  There are not any bicycle or pedestrian facilities along this roadway facility adjacent to the subject property.

SR 324 (Gravel Springs Road)

- SR 324 has recently been improved to be a four-lane divided roadway facility, from Braselton Highway to SR 20, with turn lanes and a posted speed limit of 45 mph.  SR 324 is classified as a major collector that provides a connection between the cities of Auburn and Buford.  This facility is not within a walkable distance to the proposed development.

SR 124 (Braselton Highway)

- SR 124 is an un-divided two-lane roadway facility with turn lanes and a posted speed limit of 45 mph.  SR 124 is classified as a major collector that travels in a north/south direction connecting Interstates 20 and 85.  This facility is not within a walkable distance to the proposed development.

Old Peachtree Road

- Old Peachtree Road is a two-lane undivided roadway facility with a posted speed limit of 45 mph.  This facility has been classified as a minor collector that travels in an east/west direction, just south of the subject property.  Old Peachtree Road primarily serves residential uses and the roadway has been improved with turn-lanes to improve traffic flow.  Bicycle and pedestrian facilities have been constructed in stages as new development occurs; therefore, gaps in these types of amenities exist.

Rock Springs Road

- Rock Springs Road is a two-lane undivided local roadway facility with a posted speed limit of 35 mph.  This roadway primarily serves residential uses between SR 20 and Old Peachtree Road.  The facility is located just north of the subject property and travels in an east/west direction.  Termini for this facility begin and end at Old Peachtree Road, as the roadway loops north, traversing SR 20.  Once again, bicycle and pedestrian facilities have been constructed in stages as new development occurs; therefore, gaps in these types of amenities exist.

Tech Center Parkway

- Tech Center Parkway is a two-lane local roadway constructed with a continuous center left-turn lane and a posted speed limit of 35 mph.  This facility serves both residential and non-residential land uses and is located just north of the subject property.  Bicycle and pedestrian facilities do exist along Tech Center Parkway, as development along this facility has been recently constructed.  There are sidewalks located north of the roadway and an eight-foot, multi-use path located south of the roadway.

Tech Center Drive

- Tech Center Drive is a two-lane undivided local roadway with a posted speed limit of 25 mph.  This facility serves as a connection between Rock Springs Road and Tech Center Parkway.  There are sidewalks and curbs located on both sides of the roadway.

Collins Hill Road

- Collins Hill Road is a two-lane undivided roadway facility with a posted speed limit of 40 mph.  This facility terminates at Old Peachtree Road and travels south, providing a connection to State Route 316.  Collins Hill Road mainly serves as access to numerous residential properties and school facilities within Gwinnett County.

The remaining roadways within the study area are two-lane, un-divided local roadway facilities with the exception of Horizon Drive.  Horizon Drive is classified as a minor collector and has been constructed as a four lane roadway facility with a two-way continuous left-turn lane and a posted speed limit of 45 mph.  This facility provides access to numerous office/distribution centers and provides a connection to Lawrenceville Suwanee Road, just south of Interstate 85.

  - Trip Generation

As stated earlier, the trips associated with the proposed development were estimated using the ITE Trip Generation manual (7th Edition).  In order to account for the mixture of uses anticipated for this development, ITE Trip Generation codes 230, 710, 820 and 310 were utilized.

Mixed-use reductions were calculated based upon the procedures outlined in the ITE Trip Generation Handbook, Chapter 7: Multi-Use Development.  The worksheets used in calculating these reductions for the multi-use generated trips are provided in Appendix C.  The complementary land uses within the proposed development have been inter-connected through the use of site design features that promote pedestrian accessibility and connectivity.  The daily internal capture rate and vehicle trip reductions between residential, office and retail land uses is expected to be 14%.  The internal capture rates and vehicle trip reductions for the AM and PM peak hours is expected to be 5% and 11%, respectively.

The mode split assumptions for the proposed project will be primarily the use of the single-occupant vehicle and multiple occupancy vehicles.  However, Gwinnett County does operate a local bus service, connecting neighborhoods and businesses to various cultural, shopping and educational opportunities.  It has been assumed that transit operations will take advantage of the proposed bus stops and sidewalks within the proposed development; therefore, for the purposes of this analysis approximately one percent (1%) in off-site vehicular trips was recognized for trip reduction purposes for the proposed development.

The final adjustment in trip generation was made in order to account for “pass-by” trips associated with the retail portion of the proposed development.  The pass-by trip reduction rate was calculated using the ITE Trip Generation Handbook, 5th edition.  Based upon the formula given on page I-23, a trip reduction rate of thirty-three (33) percent for the year 2012 may be assumed.  A limits test reveals that the daily volume on SR 20 within close vicinity of the Subject Property is approximately 33,060 vehicles per day.  This volume was gathered from the GDOT traffic count database.  Using the fifteen percent limits test, the total number of pass-by trips that can be realized can not exceed 5,801 vehicles for the year 2012 using a four percent (4%) average annual growth rate.

The total (net) trips generated and analyzed in this report are listed in Table 5.

| Table 5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Net Trip Generation Analysis |  |  |  |  |  |  |  |
| Reduction Factors | Daily Traffic Enter | AM Peak |  | PM Peak |  | Saturday |  |
|  |  | Enter | Exit | Enter | Exit | Enter | Exit |
| Gross Project Trips | 26,115 | 1,046 | 469 | 1,091 | 1,555 | 1,263 | 1,142 |
| Mixed-Use Reduction | -3,332 | -47 | -20 | -110 | -162 | -57 | -52 |
| Alternative Mode Reduction | -205 | -9 | -4 | -9 | -13 | -11 | -10 |
| Pass-by Reduction | -4,298 | -63 | -40 | -199 | -216 | -317 | -293 |
| Net New Trips | 18,280 | 927 | 405 | 773 | 1,164 | 878 | 787 |

  - Trip Distribution and Assignment

Vehicular trips generated by the proposed development were distributed onto the study area network using the percentages agreed upon during the methodology discussions with GRTA, ARC and Gwinnett County staff on September 22nd, 2008.  Figure 8 illustrates the anticipated residential and non-residential trip assignment percentages for the proposed development within the study area network.  These percentages were applied to the trips generated by the proposed development and the resulting volumes were assigned to the study area network.  The peak hour turning movement volumes generated by the proposed development for each peak period that was analyzed are displayed on Figures 9 through 18.

Figure 8

Figure 9

Figure 10

Figure 11

Figure 12

Figure 13

Figure 14

Figure 15

Figure 16

Figure 17

Figure 18

  - Traffic Analysis

    - Existing Traffic

The observed existing peak hour traffic volumes for the weekday AM and PM peak hours and the Saturday peak hour were input into CORSIM, version 5.1, along with optimized traffic signal cycle lengths, splits and offsets.  The existing traffic conditions, which reveal traffic control and lane configurations, have been illustrated in Figure 19.  An existing conditions analysis for the year 2008 was performed for the intersections within the study area.  It should be noted that there is not any delay associated with the entry/exit ramps to I-85 along SR 20, as these facilities are constructed as directional flow ramps.  The results from these analyses have been summarized in Table 6 and more detailed results are provided in Appendix B.  The existing peak hour traffic volumes have been illustrated in Figures 20 through 25, for each of the periods that were analyzed.

| Table 6 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Existing (2008) Intersection Level of Service Summary |  |  |  |  |  |
| Intersection | Control | LOS Standard | AM Peak (sec/veh) | PM Peak (sec/veh) | Sat Peak (sec/veh) |
| SR 20 @ Russell/Ridge Rd | Signalized | D | D (40.5) | D (44.9) | C (23.9) |
| SR 20 @ Azalea Dr (south) | Un-signalized | D | NB – B SB – A EB – B WB – B | NB – A SB – A EB – C WB – C | NB – A SB – B EB – B WB – B |
| SR 20 @ Bentbrooke Trl | Un-signalized | D | SB – A WB - A | SB – A WB – A | SB – A WB – A |
| SR 20 @ Azalea Dr (north) | Un-signalized | D | NB – A EB – D | NB – A EB – C | NB – A EB – D |
| SR 20 @ Old Peachtree Rd | Signalized | D | C (32.3) | D (45.2) | D (45.7) |
| SR 20 @ Tech Center Pkwy | Signalized | D | C (21.7) | B (12.1) | B (16.7) |
| SR 20 @ Rock Springs Rd | Signalized | D | D (37.9) | D (41.1) | D (36.7) |
| SR 20 @ signalized intersection south of I-85 | Signalized | D | A (9.7) | B (18.2) | B (13.8) |
| SR 20 @ Mall of Georgia Blvd | Signalized | D | D (35.3) | D (38.6) | D (42.6) |
| SR 20 @ Plains Court Wy | Signalized | D | A (9.7) | C (24.8) | C (28.4) |
| SR 20 @ Mall Grand Ln | Signalized | D | C (23.0) | D (35.4) | C (30.7) |
| SR 20 @ Woodward Crossing Blvd | Signalized | D | D (35.6) | C (33.9) | D (40.8) |
| SR 20 @ Gravel Springs Rd | Signalized | D | C (31.1) | C (22.1) | C (21.1) |
| Old Peachtree Rd @ Horizon Dr | Signalized | D | C (23.9) | C (22.9) | A (9.9) |
| Old Peachtree Rd @ Collins Hill Rd | Signalized | D | B (12.3) | B (16.2) | B (11.2) |
| Old Peachtree Rd @ Rock Springs Rd (west) | Un-signalized | D | SB – F EB - A | SB – B EB – A | SB – B EB – A |
| Old Peachtree Rd @ Tech Center Pkwy | Un-signalized | D | SB – A EB - A | SB – A EB – A | SB – A EB – A |
| Old Peachtree Rd @ Rock Springs Rd (east) | Un-signalized | D | SB – A EB - B | SB – C EB – A | SB – A EB – A |
| Old Peachtree Rd @ Braselton Hwy | Signalized | D | C (22.2) | C (27.6) | C (20.4) |
| Gravel Springs Rd @ Braselton Hwy | Signalized | D | D (35.7) | D (37.2) | C (26.8) |
| Tech Center Pkwy @ Tech Center Dr | Un-signalized | D | SB – A EB - A | SB – A EB – A | SB – A EB - A |
| Rock Springs Rd @ Tech Center Dr | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |

- Inadequate LOS

Figure 19

Figure 20

Figure 21

Figure 22

Figure 23

Figure 24

Figure 25

All of the intersections within the study area network currently operate at an acceptable LOS during both the weekday AM and PM peak hours and the Saturday peak hour, with the exception of the intersection located along Old Peachtree Road at Rock Springs Road (west of SR 20).  The southbound approach at this intersection currently operates at LOS F during the weekday AM peak hour.  A programmed transportation improvement that provides a traffic signal at this location has been scheduled to be completed by the year 2009.  It is not anticipated that this intersection will remain deficient for the future condition analyses.  Therefore, the No-Build and Build weekday AM and PM peak hours and the Saturday peak hour LOS standard will remain as LOS D, pursuant to GRTA’s guidelines in the Letter of Understanding.

  - 2012 No-Build Condition Traffic

No-Build traffic can be defined as future background traffic on the study area roadways not including the trips generated by the proposed DRI.  As discussed previously, the growth rates utilized for this portion of the analyses were established by GRTA.  A four percent (4%) annual growth rate was assumed for the roadways within the study area, with the exception of the following intersections, where a two and one-half percent (2.5%) annual growth rate was assumed:

- Old Peachtree Rd @ Collins Hill Rd

- Old Peachtree Rd @ Rock Springs Rd (east of SR 20)

- Old Peachtree Rd @ Braselton Hwy

- Old Peachtree Rd @ Horizon Dr

- Gravel Springs Rd @ Braselton Hwy

In addition to the general growth in background traffic, the traffic generated by the Gwinnett County Minor League Baseball Stadium was included within the 2012 No-Build traffic volumes.

The previously approved DRI for the Gwinnett County Minor League Baseball Stadium, which is under construction, will consist of a 10,000 seat stadium, along with 73,000 square feet of retail space.  Access locations for the stadium have been identified as access locations A thru E.  Access location D was omitted from this analysis as a recommended condition imposed by GRTA.  The actual design of the stadium has accounted for 2,500 off-street parking spaces.  There are a remaining 1,500 off-street parking spaces that will be obtained through parking easements upon properties within walking distance to the stadium.  There are 1,000 off-street parking spaces located at the adjacent church facility and 500 off-street parking spaces located in an office complex on Tech Center Parkway.  These access locations for additional parking opportunities were included in the future (2012) No-Build traffic conditions.

The resulting future traffic volumes that have been projected for a four year period, and the optimized signal cycle lengths, splits and offsets were input into CORSIM, version 5.1 and an analysis of the projected No-Build Conditions was performed.  The future (2012) No-Build study area network incorporated the access improvements required to serve the traffic generated by the Gwinnett County Minor League Baseball Stadium along with two programmed transportation improvements that are anticipated to be complete by the year 2012.  Each of these improvements includes:

- Intersection Improvement at SR 20 and Old Peachtree Road (Gwinnett County Project)

- Signalization of Rock Springs Road and Old Peachtree Road (west of SR 20 – Gwinnett County Project)

- Intersection Improvement at SR 20 and Tech Center Parkway (DRI #1725 Condition)

- Signalize Access B @ SR 20 (DRI #1725 Condition)

The results from this analysis have been summarized in Table 7 and more detailed results are provided in Appendix B.  These results indicate that as traffic conditions worsen, further improvements will be necessary within the study area in order for all of the intersections to operate at an acceptable LOS.  The future (2012) No-Build traffic conditions, which indicate the required traffic controls and lane configurations, have been illustrated in Figure 26, while the future (2012) No-Build traffic volumes are summarized in Figures 27 through 33.

Table 7 reveals that Azalea Drive, located south of Bentbrooke Trail, is anticipated to experience excessive delay during both the weekday peak periods that were analyzed.  Azalea Drive, north of Bentbrooke Trail is also expected to become deficient during the Saturday peak hour.  The signalized intersection along SR 20 at Russell/Ridge Road can be expected to operate at an unacceptable LOS during the weekday PM peak hour and the signalized intersection along SR 20 at Rock Springs Road is expected to become deficient during the weekday PM peak hour and during the Saturday peak hour.  A more detailed review of the results reveals that while the remaining intersections along SR 20, south of I-85, appear to operate at an acceptable LOS, upstream traffic congestion restricts traffic flows south of I-85 accounting for the acceptable LOS south of I-85. The analysis indicates that the peak flows north of I-85 will extend the peak periods beyond one hour.  In order to improve the operation of each intersection within the study area the following No-Build Condition improvements are required:

- Widen SR 20 from four (4) to six (6) lanes from Tech Center Pkwy to I-85

- SR 20 @ signalized intersection between Rock Springs Rd and I-85

  - Provide a southbound right-turn bay along SR 20

- SR 20 @ Rock Springs Rd

  - Provide a westbound, right-turn bay along Rock Springs Rd

- SR 20 @ Azalea Dr (north of Bentbrooke Trl)

  - Signalize intersection

  - Provide an eastbound, right-turn bay along Azalea Dr

- SR 20  @ Azalea Dr (south of Bentbrooke Trl)

  - Provide eastbound, right and left-turn bays along Azalea Dr

  - Provide westbound, right and left-turn bays along Azalea Dr

- SR 20 @ Russell/Ridge Rd

  - Provide an eastbound, left-turn bay along Russell Rd

  - Provide a westbound, left-turn bay along Ridge Rd

- Old Peachtree Rd @ Rock Springs Rd (east of SR 20)

  - Signalize intersection

  - Provide a southbound, right-turn bay along Rock Springs Rd

  - Provide and eastbound, left-turn bay along Old Peachtree Rd

| Table 7 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Future (2012) No-Build Condition Intersection Level of Service Summary |  |  |  |  |  |
| Without Required Improvements |  |  |  |  |  |
|  | Control | LOS Standard | AM Peak (sec/veh) | PM Peak (sec/veh) | Sat Peak (sec/veh) |
| SR 20 @ Russell/Ridge Rd | Signalized | D | C (21.5) | E (57.0) | C (24.7) |
| SR 20 @ Azalea Dr (south) | Un-signalized | D | NB – B SB – A EB – F WB - C | NB – B SB – C EB – F WB - C | NB – B SB – A EB – C WB - C |
| SR 20 @ Bentbrooke Trl | Un-signalized | D | SB – A WB - B | SB – B WB - B | SB – A WB - A |
| SR 20 @ Azalea Dr (north) | Un-signalized | D | NB – C EB - F | NB – B EB - F | NB – A EB - F |
| SR 20 @ Old Peachtree Rd | Signalized | D | B (17.7) | D (42.3) | C (31.3) |
| SR 20 @ Access A | Un-signalized | D | EB - A | EB - C | EB - A |
| SR 20 @ Access B | Signalized | D | A (4.0) | A (9.1) | B (15.2) |
| SR 20 @ Access C | Un-signalized | D | EB – A | EB - B | EB – B |
| SR 20 @ Tech Center Pkwy | Signalized | D | A – (4.2) | C (22.3) | A (6.9) |
| SR 20 @ Rock Springs Rd | Signalized | D | C (27.1) | F (84.6) | E (65.1) |
| SR 20 @ signalized intersection south of I-85 | Signalized | D | B (10.9) | C (34.9) | B (14.9) |
| SR 20 @ Mall of Georgia Blvd | Signalized | D | C (29.5) | D (47.9) | C (31.6) |
| SR 20 @ Plains Court Wy | Signalized | D | A (5.3) | B (11.6) | B (11.1) |
| SR 20 @ Mall Grand Ln | Signalized | D | A (8.8) | B (16.2) | C (27.3) |
| SR 20 @ Woodward Crossing Blvd | Signalized | D | B (17.2) | B (20.1) | C (20.8) |
| SR 20 @ Gravel Springs Rd | Signalized | D | D (36.0) | C (29.7) | C (26.8) |
| Old Peachtree Rd @ Horizon Dr | Signalized | D | B (14.0) | C (34.8) | B (14.2) |
| Old Peachtree Rd @ Collins Hill Rd | Signalized | D | B (12.7) | C (21.0) | B (13.3) |
| Old Peachtree Rd @ Rock Springs Rd (west) | Signalized | D | C (24.5) | A (6.7) | B (13.9) |
| Old Peachtree Rd @ Tech Center Pkwy | Un-signalized | D | SB – A EB - A | SB – A EB - A | SB – A EB - A |
| Old Peachtree Rd @ Rock Springs Rd (east) | Un-signalized | D | SB – B EB - B | SB – C EB - A | SB – A EB - A |
| Old Peachtree Rd @ Braselton Hwy | Signalized | D | B (14.6) | C (25.7) | B (14.4) |
| Gravel Springs Rd @ Braselton Hwy | Signalized | D | C (25.3) | C (26.0) | C (28.6) |
| Tech Center Pkwy @ Tech Center Dr/Access E | Un-signalized | D | NB – A SB – A EB – A WB - A | NB – A SB – A EB – A WB - A | NB – A SB – A EB – A WB - A |
| Rock Springs Rd @ Tech Center Dr | Un-signalized | D | NB – A WB – A | NB – A WB - A | NB – A WB - A |
| Tech Center Parkway Stadium Parking | Un-signalized | D | - | WB - A | WB - A |
| Old Peachtree Rd Stadium Parking | Un-signalized | D | - | EB - A | EB - A |

- Inadequate LOS

| Table 7, cont’d |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Future (2012) No-Build Condition Intersection Level of Service Summary |  |  |  |  |  |
| With Required Improvements |  |  |  |  |  |
|  | Control | LOS Standard | AM Peak (sec/veh) | PM Peak (sec/veh) | Sat Peak (sec/veh) |
| SR 20 @ Russell/Ridge Rd | Signalized | D | C (20.5) | C (29.3) | C (22.7) |
| SR 20 @ Azaeala Dr (south) | Un-signalized | D | NB – B SB – A EB – C WB - B | NB – A SB – B EB – D WB - C | NB – A SB – B EB – B WB - B |
| SR 20 @ Bentbrooke Trl | Un-signalized | D | SB – A WB - A | SB – B WB - B | SB – B WB - A |
| SR 20 @ Azalea Dr (north) | Signalized | D | A (4.8) | A (5.8) | A (2.4) |
| SR 20 @ Old Peachtree Rd | Signalized | D | B (18.9) | C (30.1) | C (26.61) |
| SR 20 @ Access A | Un-signalized | D | EB – A | EB – B | EB – B |
| SR 20 @ Access B | Signalized | D | A (3.6) | A (9.5) | B (10.9) |
| SR 20 @ Access C | Un-signalized | D | EB – A | EB – C | EB – A |
| SR 20 @ Tech Center Pkwy | Signalized | D | A (5.7) | B (10.9) | A (4.3) |
| SR 20 @ Rock Springs Rd | Signalized | D | B (18.5) | C (25.5) | B (19.5) |
| SR 20 @ signalized intersection south of I-85 | Signalized | D | A (8.9) | B (10.6) | A (7.6) |
| SR 20 @ Mall of Georgia Blvd | Signalized | D | C (29.2) | D (37.3) | C (28.3) |
| SR 20 @ Plains Court Wy | Signalized | D | A (5.5) | C (27.1) | B (11.3) |
| SR 20 @ Mall Grand Ln | Signalized | D | A (8.3) | B (18.6) | C (27.5) |
| SR 20 @ Woodward Crossing Blvd | Signalized | D | B (16.4) | B (10.3) | C (20.7) |
| SR 20 @ Gravel Springs Rd | Signalized | D | D (36.0) | C (35.0) | C (27.2) |
| Old Peachtree Rd @ Horizon Dr | Signalized | D | B (13.8) | D (38.1) | B (13.8) |
| Old Peachtree Rd @ Collins Hill Rd | Signalized | D | B (13.1) | D (37.1) | B (12.9) |
| Old Peachtree Rd @ Rock Springs Rd (west) | Signalized | D | C (26.8) | A (5.3) | B (12.9) |
| Old Peachtree Rd @ Tech Center Pkwy | Un-signalized | D | SB – A EB - A | SB – A EB - A | SB – A EB - A |
| Old Peachtree Rd @ Rock Springs Rd (east) | Signalized | D | A (7.9) | A (9.2) | A (5.6) |
| Old Peachtree Rd @ Braselton Hwy | Signalized | D | B (16.4) | C (24.9) | B (14.6) |
| Gravel Springs Rd @ Braselton Hwy | Signalized | D | C (25.3) | C (26.7) | C (28.8) |
| Tech Center Pkwy @ Tech Center Dr/Access E | Un-signalized | D | NB – A SB – A EB – A WB - A | NB – A SB – A EB – A WB - A | NB – A SB – A EB – A WB - A |
| Rock Springs Rd @ Tech Center Dr | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Tech Center Parkway Stadium Parking | Un-signalized | D | - | WB - A | WB - A |
| Old Peachtree Rd Stadium Parking | Un-signalized | D | - | EB - A | EB - A |

Inadequate LOS

Figure 26

Figure 27

Figure 28

Figure 29

Figure 30

Figure 31

Figure 32

Figure 33

Figure 34

Subsequent to the transportation improvements that have been previously listed, all of the intersections within the study area network operate at a LOS D or better for both the weekday AM and PM peak hours, along with the Saturday peak hour.  Additionally, all of the traffic volumes within the study area network are expected to occur within the hour that was analyzed.

Prior to any recommended transportation improvements, the eastbound approach along Azalea Drive (south of Bentbrooke Trail) along SR 20 operated at LOS F during both of the weekday peak hours.  The provision of left and right-turn bays for each of the minor approaches at this intersection improves the minor, eastbound approach during the weekday AM and PM peak hours to LOS C and D, respectively.  Providing a traffic signal at Azalea Drive (north of Bentbrooke Trail) improves the minor, eastbound approach which operated at LOS F for each period analyzed to LOS A for the same peak periods.  SR 20 at Russell/Ridge Roads operated at LOS E during the PM peak hour; however, as a result of providing eastbound and westbound left-turn bays the operation of this intersection improves to LOS C during the same peak hour.  Lastly, the intersection along SR 20 at Rock Springs Road operates at LOS F during the PM peak hour and LOS E during the Saturday peak hour without any improvements.  A westbound, right-turn bay along Rock Springs Road improves the operation of this intersection to LOS C during the weekday PM peak hour and LOS B during the Saturday peak hour.  There were no further improvements required to serve the general growth in background traffic, which includes the traffic generated by the Gwinnett County Minor League Baseball Stadium.

  - 2012 Build Condition Traffic

Lastly, the traffic generated by the proposed development was added to the future (2012) No-Build traffic volumes.  These volumes were then input into CORSIM, version 5.1 utilizing the required 2012 No-Build roadway conditions.  For the purposes of this analysis, both the weekday AM and PM peak hours, along with the Saturday peak hour conditions were analyzed.  The resulting Build Condition traffic volumes for the weekday AM and PM peak hours are displayed on Figures 35 through 39, while the Saturday Build Condition traffic volumes are displayed on Figures 40 through 44.

See Figure 20

Figure 19

The results of the intersection analysis for the future (2012) Build Condition revealed that additional roadway improvements will be needed by the year 2012, in order to accommodate the proposed development’s traffic and to acquire a LOS D at each intersection within the study area.  The results of this analysis are summarized in Table 8, and more detailed results are provided in Appendix B.  The future (2012) Build traffic conditions, which indicate the required traffic controls and lane configurations have been illustrated on Figure 45.

Figure 35

Figure 36

Figure 37

Figure 38

Figure 39

Figure 40

Figure 41

Figure 42

Figure 43

Figure 44

| Table 8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Future (2012) Build Condition Intersection Level of Service Summary |  |  |  |  |  |
| Without Required Improvements |  |  |  |  |  |
|  | Control | LOS Standard | AM Peak (sec/veh) | PM Peak (sec/veh) | Sat Peak (sec/veh) |
| SR 20 @ Russell/Ridge Rd | Signalized | D | C (21.5) | C (34.0) | C (24.2) |
| SR 20 @ Azalea Dr (south) | Un-signalized | D | NB – B SB – B EB – C WB – C | NB – A SB – B EB – F WB – E | NB – A SB – B EB – B WB – C |
| SR 20 @ Bentbrooke Trl | Un-signalized | D | SB – B WB - C | SB – B WB – C | SB – B WB - C |
| SR 20 @ Azalea Dr (north) | Signalized | D | A (4.2) | A (6.4) | A (2.7) |
| SR 20 @ Old Peachtree Rd | Signalized | D | C (21.9) | E (62.6) | D (38.4) |
| SR 20 @ Access A | Un-signalized | D | EB - A | EB - F | EB - C |
| SR 20 @ Access B | Signalized | D | A (5.7) | C (28.5) | B (19.6) |
| SR 20 @ Access C | Un-signalized | D | EB - A | EB - F | EB - C |
| SR 20 @ Tech Center Pkwy | Signalized | D | A (6.3) | B (15.4) | A (8.2) |
| SR 20 @ Rock Springs Rd | Signalized | D | C (21.7) | C (27.5) | C (20.2) |
| SR 20 @ signalized intersection south of I-85 | Signalized | D | A (9.7) | B (10.5) | A (8.1) |
| SR 20 @ Mall of Georgia Blvd | Signalized | D | C (29.5) | C (27.6) | D (41.8) |
| SR 20 @ Plains Court Wy | Signalized | D | A (5.4) | A (9.9) | C (25.4) |
| SR 20 @ Mall Grand Ln | Signalized | D | A (9.0) | B (19.6) | C (27.7) |
| SR 20 @ Woodward Crossing Blvd | Signalized | D | B (15.5) | B (10.3) | B (20.0) |
| SR 20 @ Gravel Springs Rd | Signalized | D | D (35.6) | C (34.9) | C (28.6) |
| Old Peachtree Rd @ Horizon Dr | Signalized | D | B (14.8) | F (93.8) | C (29.1) |
| Old Peachtree Rd @ Collins Hill Rd | Signalized | D | B (13.8) | D (40.9) | B (13.5) |
| Old Peachtree Rd @ Rock Springs Rd (west) | Signalized | D | C (21.5) | A (6.7) | B (18.2) |
| Old Peachtree Rd @ Tech Center Pkwy | Un-signalized | D | SB – A EB - A | SB – A EB – A | SB – A EB - A |
| Old Peachtree Rd @ Rock Springs Rd (east) | Signalized | D | A (7.6) | B (11.5) | A (6.8) |
| Old Peachtree Rd @ Braselton Hwy | Signalized | D | C (20.8) | C (34.5) | B (17.9) |
| Gravel Springs Rd @ Braselton Hwy | Signalized | D | C (25.1) | C (27.4) | C (26.2) |
| Tech Center Pkwy @ Tech Center Dr/Access E | Un-signalized | D | NB – A SB – A EB – A WB - A | NB – B SB – A EB – A WB - A | NB – B SB – B EB – A WB - A |
| Rock Springs Rd @ Tech Center Dr | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Tech Center Parkway Stadium Parking/Access I | Un-signalized | D | SB – A EB - A | SB – A EB – A WB - A | SB – A EB – A WB - A |
| Old Peachtree Rd Stadium Parking | Un-signalized | D | - | EB - B | EB - A |
| Tech Center Dr @ Access F | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Rock Springs Rd @ Access G | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Rock Springs Rd @ Access H | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Old Peachtree Rd @ Access J | Un-signalized | D | SB – A EB - A | SB – A EB - A | SB – A EB - B |

- Inadequate LOS

| Table 8, cont’d |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Future (2012) Build Condition Intersection Level of Service Summary |  |  |  |  |  |
| With Required Improvements |  |  |  |  |  |
|  | Control | LOS Standard | AM Peak (sec/veh) | PM Peak (sec/veh) | Sat Peak (sec/veh) |
| SR 20 @ Russell/Ridge Rd | Signalized | D | C (22.0) | D (36.3) | C (24.6) |
| SR 20 @ Azalea Dr (south) | Signalized | D | A (6.7) | B (15.4) | A (4.5) |
| SR 20 @ Bentbrooke Trl | Un-signalized | D | SB – B WB – C | SB – C WB – C | SB – B WB – C |
| SR 20 @ Azalea Dr (north) | Signalized | D | A (5.9) | B (19.0) | A (2.9) |
| SR 20 @ Old Peachtree Rd | Signalized | D | B (17.7) | C (35.0) | C (25.9) |
| SR 20 @ Access A | Un-signalized | D | EB - A | EB – B | EB - A |
| SR 20 @ Access B | Signalized | D | A (4.9) | B (15.6) | C (22.1) |
| SR 20 @ Access C | Un-signalized | D | EB - A | EB – B | EB – A |
| SR 20 @ Tech Center Pkwy | Signalized | D | A (4.3) | B (14.6) | B (14.6) |
| SR 20 @ Rock Springs Rd | Signalized | D | C (21.2) | C (27.3) | C (22.2) |
| SR 20 @ signalized intersection south of I-85 | Signalized | D | A (9.5) | B (12.3) | A (8.4) |
| SR 20 @ Mall of Georgia Blvd | Signalized | D | C (29.8) | D (37.5) | D (45.7) |
| SR 20 @ Plains Court Wy | Signalized | D | A (5.9) | C (25.8) | C (34.8) |
| SR 20 @ Mall Grand Ln | Signalized | D | A (9.3) | C (20.5) | B (17.6) |
| SR 20 @ Woodward Crossing Blvd | Signalized | D | B (15.6) | B (11.5) | C (21.9) |
| SR 20 @ Gravel Springs Rd | Signalized | D | D (37.0) | C (31.0) | C (28.2) |
| Old Peachtree Rd @ Horizon Dr | Signalized | D | B (13.7) | C (27.3) | C (21.8) |
| Old Peachtree Rd @ Collins Hill Rd | Signalized | D | B (13.3) | C (20.5) | B (13.7) |
| Old Peachtree Rd @ Rock Springs Rd (west) | Signalized | D | C (25.2) | A (4.7) | C (20.8) |
| Old Peachtree Rd @ Tech Center Pkwy | Un-signalized | D | SB – A EB – A | SB – A EB – A | SB – A EB – A |
| Old Peachtree Rd @ Rock Springs Rd (east) | Signalized | D | A (6.7) | B (14.2) | A (6.2) |
| Old Peachtree Rd @ Braselton Hwy | Signalized | D | C (21.1) | C (29.9) | B (17.8) |
| Gravel Springs Rd @ Braselton Hwy | Signalized | D | C (25.4) | C (27.5) | C (25.3) |
| Tech Center Pkwy @ Tech Center Dr/Access E | Un-signalized | D | NB – A SB – A EB – A WB - A | NB – B SB – A EB – A WB - A | NB – A SB – B EB – A WB - A |
| Rock Springs Rd @ Tech Center Dr | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Tech Center Parkway Stadium Parking/Access I | Un-signalized | D | SB – A EB - A | SB – A EB – A WB – A | SB – A EB – A WB – A |
| Old Peachtree Rd Stadium Parking | Un-signalized | D | - | EB - A | EB - A |
| Tech Center Dr @ Access F | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Rock Springs Rd @ Access G | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Rock Springs Rd @ Access H | Un-signalized | D | NB – A WB - A | NB – A WB - A | NB – A WB - A |
| Old Peachtree Rd @ Access J | Un-signalized | D | SB – A EB - A | SB – A EB - A | SB – A EB - A |

Figure 21

Figure 45

Table 8 indicates that the following intersections are anticipated to become deficient by the Build year (2012) during the weekday PM peak hour:

- SR 20 @ Azalea Dr (south of Bentbrooke Trl)

- SR 20 @ Old Peachtree Rd

- SR 20 @ Access A

- SR 20 @ Access C

- Old Peachtree Rd @ Horizon Dr

Once again, a more detailed review of the results indicate that all of the traffic volumes within the study area network are not served within the peak hour that was analyzed; therefore, the peak period can be expected to be extended beyond one hour.  In order for all of the intersections within the study area top operate at an acceptable LOS (LOS D or better) the following improvements are required for the Build Condition:

- Widen SR 20 from four (4) to six (6) lanes from Azalea Dr (north of Bentbrooke Trl) to Tech Center Pkwy

- SR 20 @ Tech Center Pkwy

  - Provide a southbound right-turn bay along SR 20

- SR 20 @ Old Peachtree Rd

  - Extend the eastbound, left-turn bay along Old Peachtree Rd

  - Convert existing southbound right-turn bay to a through movement and provide a southbound right-turn bay along SR 20

- SR 20 @ Azalea Dr (south of Bentbrooke Trl)

  - Signalize intersection

- Old Peachtree Rd @ Collins Hill Rd

  - Extend eastbound right-turn bay

As a result of these improvements, all of the intersections within the study area operate at an acceptable LOS for each of the peak periods that were analyzed.  Additionally, the traffic volumes that are projected for the year 2012, with the construction of the proposed development, are served within the one hour time periods that were analyzed.  The intersection located along SR 20 at Azalea Drive, south of Bentbrooke Trail, is anticipated to operate at LOS B during the PM peak hour with the required improvements.  This improves the operation of the minor street approaches along Azalea Drive, which operated at LOS F traveling eastbound during the weekday PM peak hour and LOS E traveling westbound during the same peak hour without the recommended transportation improvements.  The additional capacity provided along SR 20 from Azalea Drive, north of Bentbrooke Trail, to Tech Center Parkway works to improve the operation of the intersections along SR 20 at Old Peachtree Road, Access A and Access C.  SR 20 and Old Peachtree Road was improved from LOS E during the weekday PM peak hour to LOS C during the same peak hour.  The access locations along SR 20 at Access A and Access C were each improved from LOS F during the weekday PM peak hour to LOS A during the same peak hour.  Lastly, the intersection along Old Peachtree Road at Horizon Drive was improved from LOS F to LOS C during the weekday PM peak hour by improving the operation of an adjacent intersection.  There was no further transportation improvements required to serve the traffic generated by the proposed development.

  - Queue Analysis

Pursuant to GRTA’s recommendation, a queue analysis was performed for the intersection along SR 20 at Tech Center Parkway.  The results of this analysis are provided below in Table 9.  Subsequent to the provision of the required transportation improvements for the Build Condition, the lengths of each approach queue do not exceed the recommended turn-bay lengths.  The recommended northbound left-turn bay length for the Build Condition is 250 feet.  The recommended southbound right-turn bay length is 200 feet for the Build Condition.  Lastly, the eastbound approach consists of two lanes supplemented with a 150-foot left-turn bay.  One lane is channelized for right-only turn movements while another is channelized for left-only turn movements, thereby providing for a dual left-turn movement to SR 20 northbound.

| Table 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Queue Analysis |  |  |  |  |
| Existing (2008) Weekday AM Peak Hour |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 0’ | 75’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 0’ |
|  | Eastbound | Left | 25’ | 75’ |
|  |  | Right | 0’ | 25’ |
| Existing (2008) Weekday PM Peak Hour |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 0’ | 50’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 25’ |
|  | Eastbound | Left | 25’ | 125’ |
|  |  | Right | 0’ | 75’ |
| Existing (2008) Saturday Peak Hour |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 0’ | 25’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 25’ |
|  | Eastbound | Left | 25’ | 100’ |
|  |  | Right | 0’ | 25’ |

| Table 9, cont’d |  |  |  |  |
| --- | --- | --- | --- | --- |
| Queue Analysis |  |  |  |  |
| Future (2012) No-Build Weekday AM Peak Hour (without required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 0’ | 50’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 25’ |
|  | Eastbound | Left | 25’ | 100’ |
|  |  | Right | 0’ | 75’ |
| Future (2012) No-Build Weekday PM Peak Hour (without required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 25’ | 200’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 75’ |
|  | Eastbound | Left | 50’ | 150’ |
|  |  | Right | 0’ | 75’ |
| Future (2012) No-Build Saturday Peak Hour (without required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 25’ | 175’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 25’ |
|  | Eastbound | Left | 25’ | 125’ |
|  |  | Right | 0’ | 50’ |

| Table 9, cont’d |  |  |  |  |
| --- | --- | --- | --- | --- |
| Queue Analysis |  |  |  |  |
| Future (2012) No-Build Weekday AM Peak Hour (with required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 0’ | 50’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 175’ |
|  | Eastbound | Left | 25’ | 75’ |
|  |  | Right | 0’ | 75’ |
| Future (2012) No-Build Weekday PM Peak Hour (with required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 50’ | 150’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 25’ | 250’ |
|  | Eastbound | Left | 25’ | 150’ |
|  |  | Right | 0’ | 50’ |
| Future (2012) No-Build Saturday Peak Hour (with required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 25’ | 200’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 200’ |
|  | Eastbound | Left | 25’ | 125’ |
|  |  | Right | 0’ | 50’ |

| Table 9, cont’d |  |  |  |  |
| --- | --- | --- | --- | --- |
| Queue Analysis |  |  |  |  |
| Future (2012) Build Weekday AM Peak Hour (without required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 25’ | 100’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 175’ |
|  | Eastbound | Left | 25’ | 100’ |
|  |  | Right | 0’ | 50’ |
| Future (2012) Build Weekday PM Peak Hour (without required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 75’ | 300’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 75’ | 350’ |
|  | Eastbound | Left | 75’ | 225’ |
|  |  | Right | 25’ | 150’ |
| Future (2012) Build Saturday Peak Hour (without required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 75’ | 325’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 250’ |
|  | Eastbound | Left | 50’ | 175’ |
|  |  | Right | 0’ | 50’ |

| Table 9, cont’d |  |  |  |  |
| --- | --- | --- | --- | --- |
| Queue Analysis |  |  |  |  |
| Future (2012) Build Weekday AM Peak Hour (with required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 0’ | 100’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 50’ |
|  | Eastbound | Left | 25’ | 100’ |
|  |  | Right | 0’ | 50’ |
| Future (2012) Build Weekday PM Peak Hour (with required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 50’ | 200’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 75’ |
|  | Eastbound | Left | 50’ | 225’ |
|  |  | Right | 0’ | 125’ |
| Future (2012) Build Saturday Peak Hour (with required improvements) |  |  |  |  |
| Intersection | Approach | Movement | Average Queue | Maximum Queue |
| SR 20 @ Tech Center Pkwy | Northbound | Left | 25’ | 200’ |
|  |  | Thru | - | - |
|  | Southbound | Thru | - | - |
|  |  | Right | 0’ | 150’ |
|  | Eastbound | Left | 50’ | 225’ |
|  |  | Right | 0’ | 75’ |

  - Identification of Programmed Projects

Based on information received from Gwinnett County, the Atlanta Regional Commission and GDOT, there are three roadway enhancement projects within vicinity of the proposed project that are planned for construction by the build year, 2012.  These programmed transportation improvement projects have been incorporated into the future (2012) analyses and correspondence verifying completion dates and costs are provided in Appendix D.  These projects include two (2) intersection improvements and one (1) corridor improvement along SR 20.  These projects are summarized in Table 10 and a project description for each follows.  The locations of these projects are displayed on Figure 46.

| Table 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Programmed Transportation Improvements |  |  |  |  |  |  |  |
| Project No. | Source | Sponsor | Description | Location |  | Build Year | Cost |
|  |  |  |  | from | to |  |  |
| M-0533 | SPLOST | Gwinnett | Intersection Improvement | Old Peachtree Rd @ SR 20 |  | 2009 | $1,600,000 |
| M-0642 | SPLOST | Gwinnett | Intersection Improvement | Old Peachtree Rd @ Rock Springs Rd |  | 2009 | $120,000 |
| M-0036-5 | Gwinnett | Gwinnett | ATMS | Lawrenceville | I-985 | 2009 | unknown |

Gwinnett County Project #M-0533

- Intersection improvement at the intersection of SR 20 and Old Peachtree Road - This project will include dual left-turn lanes from SR 20 southbound onto Old Peachtree Road and additional lanes on Old Peachtree Road, improving the overall efficiency of the intersection.  Land acquisition for this project began in February 2008 and the project construction will be complete prior to the opening of the proposed development.

Gwinnett County Project #M-0642

- Intersection improvement at the intersection of Old Peachtree Road and Rock Springs Road – Gwinnett County has identified the need for a traffic signal at this location and construction will be complete prior to the opening of the proposed development.

Gwinnett County Project #M-0036-5

- The County is currently constructing a project to install fiber optic communications cable from the Gwinnett Traffic Control Center in Lawrenceville along the SR 20 Corridor to the I-985 interchange.  This project will provide enhanced signal timing within the study area utilizing traffic monitoring devices between Old Peachtree Road and I-85.  These devices will allow Gwinnett County to make any necessary adjustments during events at the proposed minor league baseball stadium.

Figure 46

  - Ingress/Egress Analysis

Vehicular access to the proposed development has been proposed at nine (9) locations.  All of these access locations operate as full-movement ingress/egress with the exception to two (2) access locations that are located along SR 20.  These two access locations were previously approved in DRI # 1725, as right-in/right-out access points.  The locations for each of these access driveways were previously illustrated on Figure 5.

Access locations A and C are planned to be constructed as right-in/right-out driveways with single lanes both entering and exiting.  Driveway B, which requires signalization, shall consist of two eastbound egress lanes designated as left-turn only and shall be further supplemented with a right-turn bay.  Westbound ingress at Driveway B shall consist of two lanes to accommodate inbound traffic from SR 20.  Driveway E does not require signalization; however, the northbound egress approach at this location shall be constructed with two approach lanes.  The first lane shall be designated as a right-turn only movement, while the second lane shall be designated as a through only movement.  This egress approach shall be further supplemented with a left-turn bay.  Southbound ingress at Driveway E shall consist of two approach lanes for both surface area and structured parking.

The remaining access locations for the proposed development have not been previously reviewed by GRTA.  Each of these remaining access locations, with the exception of Access I, have been proposed to be constructed with one ingress lane and one egress lane.  Access I has been proposed to include a left-turn bay for traffic exiting the proposed development.  In order to avoid conflicting left-turn movements, Access I should be relocated to be directly aligned with an access location that serves office uses south of Tech Center Parkway.  The remaining access locations work to disperse and mitigate any traffic impacts within the study area as stop-sign controlled intersections, serving both the residential and non-residential components for the entire development.

  - Internal Circulation Analysis

The proposed development has been designed to enhance internal circulation.  Specific design features that promote efficient on-site pedestrian movements include sidewalks and parking inter-connectivity.  There are not any features that result in inefficient on-site pedestrian movements.  The site plan provided, in this document, includes sidewalks within the proposed site and along adjacent roadways.  Pedestrian path distances have been minimized within the parking areas and all future complimentary land uses will be accessible by pedestrians.  The transportation analyses have revealed that all entrances to the parking facilities operate at an acceptable LOS, with the recommended Build Condition improvements.  Any conflicting vehicle-pedestrian movements within the proposed development should be addressed through appropriate signage and striping.  Efficient vehicle movements have been promoted through inter-connectivity and appropriate driveway alignments.

  - Compliance with Comprehensive Plan Analysis

The “Future Land Use Plan” for Gwinnett County printed on March 7, 2006 by the Gwinnett County Department of Planning and Development designates the majority of the subject property as a commercial land use.  The proposed development provides a mixture of uses.  Compliance with the Gwinnett County 2020 Land Use Plan will require an amendment to the Future Land Use Plan.  As indicated previously, the applicant intends to obtain a “MUO” zoning classification that allows for the construction of each use within a mixed-use overlay district.  The future land use classifications for the subject property and for the adjoining properties are illustrated on Figure 47.

Figure 47

  - DRI Review Checklist (Non-Expedited Review Criteria)

  - General Information

  - Regional Policies and Adopted Plans

  - Environmental

  - Project

  - Non-Expedited Transportation Analysis

  - ARC Score Sheet

Development of Regional Impact #1950

Gwinnett Stadium Mixed-Use

Gwinnett County, Georgia

Appendix

Appendix A

Data Collection

Appendix B

Level of Service Worksheets

Appendix C

Trip Generation Worksheets

Appendix D

Programmed Transportation Improvements